Summary of Attacks

Attack Scenarios

Deployment-grounded controlled-attack catalogue, tailored to the testbed's own devices. Categories follow the controlled-attack session labels used by the Home Assistant marker scheme (scan, web, brute_force, cve_exploit, c2, dos/ddos, exfil, mitm, lateral). Techniques are mapped to MITRE ATT&CK (Enterprise) and the Lockheed Martin Cyber Kill Chain. All attacks are executed only against the author's own devices in an isolated, institutionally approved testbed from the adversary LAN (10.21.40.58-.62); no third party is targeted, and no live self-propagating malware is deployed

Kill-chain linkage

- Reconnaissance: Port & vulnerability scanning (scan)

- Weaponization: Exploit/payload preparation, done offline on the attacker host

- Delivery: Positioning for MITM; delivery of the exploit/request to the exposed service

- Exploitation: Web attacks, CVE exploitation, successful brute force

- Installation: Web shell / backdoor / implant placed on the compromised device

- Command & Control: Botnet/C2 beaconing (c2)

- Actions on Objectives: DoS/DDoS, data exfiltration, MITM interception, lateral movement

Table 1. Attack category, technique, tool, ATT&CK mapping and kill-chain phase

| Category (session label) | Technique / sub-type | Tool(s) | MITRE Tactic | MITRE Technique (ID) | Kill-chain phase |
| --- | --- | --- | --- | --- | --- |
| Port & Vuln Scanning (scan) | Host discovery / ping sweep | nmap -sn; fping | Reconnaissance | Active Scanning: Scanning IP Blocks (T1595.001) | Reconnaissance |
| Port & Vuln Scanning (scan) | Port scan (service/version) | nmap -sS -sV; masscan | Discovery | Network Service Discovery (T1046) | Reconnaissance |
| Port & Vuln Scanning (scan) | OS / firmware fingerprinting | nmap -O | Reconnaissance | Active Scanning (T1595) | Reconnaissance |
| Port & Vuln Scanning (scan) | Vulnerability scan | nmap --script vuln; Nuclei; OpenVAS | Reconnaissance | Active Scanning: Vulnerability Scanning (T1595.002) | Reconnaissance |
| Web Attacks (web) | Cross-site scripting (XSS) | OWASP ZAP; Burp Suite | Initial Access | Exploit Public-Facing Application (T1190) | Exploitation |
| Web Attacks (web) | SQL injection | sqlmap | Initial Access | Exploit Public-Facing Application (T1190) | Exploitation |
| Web Attacks (web) | Command injection | Burp; manual | Initial Access / Execution | Exploit Public-Facing App (T1190); Command & Scripting Interpreter (T1059) | Exploitation |
| Web Attacks (web) | Auth bypass / path traversal / misconfig | nikto; Burp | Initial Access | Exploit Public-Facing Application (T1190) | Exploitation |
| Web Attacks (web) | Malicious upload / web shell | Burp; manual | Persistence | Server Software Component: Web Shell (T1505.003) | Installation |
| Credential Brute-Force (brute_force) | Dictionary brute force (SSH/Telnet/HTTP/RTSP/ONVIF) | hydra; medusa; ncrack | Credential Access | Brute Force: Password Guessing (T1110.001) | Exploitation |
| Credential Brute-Force (brute_force) | Credential stuffing (reused creds) | hydra | Credential Access | Brute Force: Credential Stuffing (T1110.004) | Exploitation |
| CVE Exploitation (cve_exploit) | CVE-2026-1871 - Tapo C200 RTSP auth buffer overflow -> DoS | Metasploit / custom PoC | Initial Access / Impact | Exploit Public-Facing Application (T1190); Endpoint DoS (T1499) | Exploitation / Actions on Objectives |
| CVE Exploitation (cve_exploit) | CVE-2020-5735 - Amcrest port 37777 buffer overflow -> DoS/RCE | Metasploit / custom PoC | Initial Access / Execution | Exploit Public-Facing Application (T1190) | Exploitation |
| CVE Exploitation (cve_exploit) | CVE-2023-38907 - Tapo L530E session-message replay | bettercap + custom replay | Credential Access / Collection | Exploitation for Credential Access (T1212); replay (T1557) | Exploitation |
| CVE Exploitation (cve_exploit) | CVE-2021-35067 - Meross MSG100 capture-replay auth bypass | bettercap + custom replay | Initial Access / Impact | Adversary-in-the-Middle (T1557); replay to actuate | Exploitation / Actions on Objectives |
| Botnet / C2 (c2) | C2 beaconing from a controlled implant on a compromised device | Sliver / Covenant / scripted beacon | Command and Control | Application Layer Protocol (T1071); Non-Standard Port (T1571) | Command & Control |
| DoS / DDoS (dos, ddos) | Volumetric flood (SYN/UDP/ICMP) | hping3 | Impact | Network DoS: Direct Network Flood (T1498.001) | Actions on Objectives |
| DoS / DDoS (dos, ddos) | Connection / application exhaustion (SYN / HTTP / slow) | hping3; golang-httpflood; slowloris | Impact | Endpoint DoS (T1499 / .001 / .003) | Actions on Objectives |
| Data Exfiltration (exfil) | Exfil from a compromised device to attacker endpoint | scp/curl; DNS tunnel (iodine) | Exfiltration | Exfiltration Over C2 Channel (T1041); Over Alternative Protocol (T1048) | Actions on Objectives |
| MITM (mitm) | ARP spoofing + sniffing | ettercap; bettercap | Collection / Credential Access | AiTM: ARP Cache Poisoning (T1557.002); Network Sniffing (T1040) | Delivery / Actions on Objectives |
| MITM (mitm) | DNS spoofing / redirect | ettercap; bettercap | Collection | Adversary-in-the-Middle (T1557) | Delivery / Actions on Objectives |
| Lateral Movement (lateral) | Pivot from a compromised device to another device | Metasploit pivot; manual | Lateral Movement | Exploitation of Remote Services (T1210); Remote Services (T1021) | Exploitation / Lateral Movement |
| Lateral Movement (lateral) | Internal recon from the pivot host | nmap (from compromised host) | Discovery | Network Service Discovery (T1046) | Lateral Movement |

Devices Inventory

| Device Model | Private IP | Wire guard address | Mac Address | Firmware Version |
| --- | --- | --- | --- | --- |
| Shelly Plug S Gen3 | 10.21.30.39 | 10.100.101.39 | 8c:bf:ea:a3:aa:a0 | 20240820-134301/ 1.2.3-plugsg3prod0-gec79607 |
| H6008 E27 | 10.21.30.33 | 10.100.101.33 | 5C:E7:53:6D:9A:06 | V1.01.25 |
| ZY-M100 Motion Sensor | 10.21.30.14 | 10.100.101.14 | 4C:A9:19:B3:5D:3F | Main Module: V2.7.7 MCU: V1.0.0 |
| Piotek_EspTempsensor | 10.21.30.11 | 10.100.101.11 | d4:8c:49:33:2a:d1 | V2026.3.1 |
| Alantop Smart Desk Lamp RGBCW2 | 10.21.30.24 | 10.100.101.24 | 80:64:7C:D9:91:59 | Main Module: V1.0.0 MCU: V2.1.0 |
| Tapo P125M Fan Plug | 10.21.30.37 | 10.100.101.37 | 98:25:4a:cb:ed:96 | 1.2.3 Build 240617 Rel.153525 |
| Govee H714E Humidifier | 10.21.30.31 | 10.100.101.31 | 3C:DC:75:0E:06:94 | V1.00.33 |
| Tapo L530 light | 10.21.30.41 | 10.100.101.41 | e0:d3:62:c0:06:f3 | 1.1.9 Build 240524 Rel.144922 |
| Tapo P125M Plug | 10.21.30.38 | 10.100.101.38 | 5c:e9:31:c4:73:79 | 1.2.3 Build 240617 Rel.153525 |
| PIR Motion Sensor-P01 | 10.21.30.17 | 10.100.101.17 | F8:17:2D:29:3B:16 | Main Module: V2.1.12 MCU: V1.0.0 |
| Piotek_EspTempsensor | 10.21.30.21 | 10.100.101.21 | 40:91:51:68:8c:5e | V2026.3.1 |
| Tapo L530 light | 10.21.30.34 | 10.100.101.34 | e0:d3:62:bf:f7:a1 | 1.1.9 Build 240524 Rel.144922 |
| Tapo C200 | 10.21.30.10 | 10.100.101.10 | ec:75:0c:4e:e9:a6 | V 1.3.3 |
| TP-L900 Light Strip | 10.21.30.26 | 10.100.101.26 | c0:3a:55:4f:1c:d3 | V1.1.1 build 240625 Rel.205528 |
| PIR Motion Sensor-P01 | 10.21.30.18 | 10.100.101.18 | F8:17:2D:29:33:B1 | Main Module: V2.1.12 MCU: V1.0.0 |
| Tapo L530 light | 10.21.30.44 | 10.100.101.44 | ac:a7:f1:8c:e6:74 | 1.1.9 Build 240524 Rel.144922 |
| PIR Motion Sensor-P01 | 10.21.30.9 | 10.100.101.9 | F8:17:2D:29:33:47 | Main Module: V2.1.12 MCU: V1.0.0 |
| TC70 | 10.21.30.19 | 10.100.101.19 | 8c:86:dd:5f:8c:e2 | V1.0.17 |
| Tapo L530 light | 10.21.30.43 | 10.100.101.43 | ac:a7:f1:8c:e4:3d | 1.1.9 Build 240524 Rel.144922 |
| PIR Motion Sensor-P01 | 10.21.30.15 | 10.100.101.15 | 38:A5:C9:B7:E8:EE | Main Module: V2.1.12 MCU: V1.0.0 |
| Shelly2PM Gen4 | 10.21.30.40 | 10.100.101.40 | cc:ba:97:df:68:88 | 20260710-101230/2.0.0-g87fbfa4 |
| Reolink E1 | 10.21.30.20 | 10.100.101.20 | ec:71:db:fd:6b:6d | V3.1.0.4417_2412122130 |
| Govee H714E Humidifier | 10.21.30.30 | 10.100.101.30 | 3C:DC:75:0A:80:54 | V1.00.33 |
| Tapo L530 light | 10.21.30.45 | 10.100.101.45 | ac:a7:f1:8c:e6:36 | 1.1.9 Build 240524 Rel.144922 |
| PIR Motion Sensor-P01 | 10.21.30.16 | 10.100.101.16 | 38:A5:C9:B7:E9:B7 | Main Module: V2.1.12 MCU: V1.0.0 |
| Alexa Echo Dot Gen 4 | 10.21.30.28 | 10.100.101.28 | B4:B7:42:56:4A:24 | V13222531716 |
| Tapo L530 light | 10.21.30.42 | 10.100.101.42 | e0:d3:62:c0:15:14 | 1.1.9 Build 240524 Rel.144922 |
| Govee H714E Humidifier | 10.21.30.32 | 10.100.101.32 | 3C:DC:75:0A:E1:50 | V1.00.33 |
| Alantop Smart Desk Lamp RGBCW2 | 10.21.30.25 | 10.100.101.25 | 80:64:7C:D9:AF:CF | Main Module: V1.0.0 MCU: V2.1.0 |
| Tapo P100 Fan Plug | 10.21.30.36 | 10.100.101.36 | b4:b0:24:44:d3:e2 | 1.5.5 Build 20230927 Rel.40646 |
| TP P115 Lamp Plug | 10.21.30.35 | 10.100.101.35 | 6c:4c:bc:d9:64:e9 | V1.4.8 Build 260804 Rel. 045837 |
| ZY-M100 Motion Sensor | 10.21.30.13 | 10.100.101.13 | 38:A5:C9:3D:B4:26 | Main Module: V2.7.7 MCU: V1.0.0 |
| Piotek_EspTempsensor | 10.21.30.23 | 10.100.101.23 | d4:8c:49:33:2a:e7 | V2026.3.1 |
| TP-L900 Light Strip | 10.21.30.27 | 10.100.101.27 | c0:3a:55:3b:5d:54 | V1.1.1 build 240625 Rel.205528 |
| Amcrest IP2M-841B-V3 | 10.21.30.12 | 10.100.101.12 | 34:46:63:20:49:EF | Web Version: V3.2.1.18144 ONVIF Version 21.12(V3.1.0.1207744) |
| Amazon FireTv Stick 4K | 10.21.30.57 | 10.100.101.57 | 54:2B:1C:F1:B0:47 | Fire OS 8.1.8.2(RS8182/3811) FireTV Home V7313002.1 |
| Reolink D340W | 10.21.30.56 | 10.100.101.56 | 7C:01:3E:9C:D0:BA | Build 2508131282 Config V3.0.0.0 Firmware V3.0.0.0.4662_2508131282 |
| Meros MSG100 | 10.21.30.55 | 10.100.101.55 | C4:E7:AE:15:7C:57 | V4.2.20 |
| H5179 Humidity Sensor | 10.21.30.54 | 10.100.101.54 | 5C:E7:53:18:F0:9A | V1.00.11 |
| H5179 Humidity Sensor | 10.21.30.53 | 10.100.101.53 | 5C:E7:53:16:F8:74 | V1.00.11 |
| H5179 Humidity Sensor | 10.21.30.52 | 10.100.101.52 | 5C:E7:53:13:1D:BA | V1.00.11 |
| Sonoff DW2 | 10.21.30.51 | 10.100.101.51 | D0:27:04:85:58:B6 | V1000.2.1130 |
| SwitchBot Wate Leak Detector | 10.21.30.50 | 10.100.101.50 | 64:E8:33:A2:F7:4C | V1.4 |
| Beok TOL47 Thermostart | 10.21.30.49 | 10.100.101.49 | 00:33:7A:B6:E9:6B | Main Module: V1.0.0(3.1.42) MCU: V1.0.0 |
| HBN Plug | 10.21.30.48 | 10.100.101.48 | D8:D6:68:AE:76:9C | V1.1.17 |
| HBN Plug | 10.21.30.47 | 10.100.101.47 | D8:D6:68:AE:75:9A | V1.1.17 |

Table 2. CVEs Detail Description

| CVE | Device (testbed) | Vulnerability type | How it is exploited | Impact | CVSS | ATT&CK / kill-chain |
| --- | --- | --- | --- | --- | --- | --- |
| CVE-2026-1871 | Tapo C200 (v5 < 1.4.4) | Stack-based buffer overflow in RTSP authentication (Authorization header) | Crafted RTSP auth request crashes the RTSP service and forces a reboot | Denial of Service | 7.1 (High) | T1190 -> T1499 / Actions on Objectives |
| CVE-2020-5735 | Amcrest camera / NVR | Stack-based buffer overflow over port 37777 (authenticated) | Authenticated remote attacker sends a crafted request on 37777 | DoS and possible RCE | 8.8 (High) | T1190 / Exploitation |
| CVE-2023-38907 | Tapo L530 / L530E (< 1.2.4) | Message replay: replay of messages encrypted with a still-valid session key | Replay previously captured session messages (requires prior sniffing/MITM) | Unauthorised control / credential disclosure | High | T1212 / T1557 / Exploitation |
| CVE-2021-35067 | Meross MSG100 garage opener (< 3.2.3) | Capture-replay authentication bypass (CWE-294) | Replay/modify a sniffed command (e.g., turn a Close into an Open) | Unauthorised actuation (open/close) | 8.1 (High) | T1557 / Exploitation -> Actions on Objectives |

Table 3. CVEs

| Device | Firmware | CVE | Description |
| --- | --- | --- | --- |
| Tapo C200 | 1.4.4:build_250922 | CVE-2025-14300 | The HTTPS service on Tapo C200 v3, v5, C425 v1.2 and C100 v5 exposes a connectAP interface without proper authentication. An unauthenticated attacker on the same local network segment can exploit this to modify the device’s Wi-Fi configuration, resulting in loss of connectivity and denial-of-servic |
| Tapo C200 | V1.1.15 | CVE-2021-4045 | TP-Link Tapo C200 IP camera, on its 1.1.15 firmware version and below, is affected by an unauthenticated RCE vulnerability, present in the uhttpd binary running by default as root. The exploitation of this vulnerability allows an attacker to take full control of the camera. |
| Tapo C200 | 1.3.5:build_260228 | CVE-2026-1871 | TP-Link Tapo C200 v5 contains a stack-based buffer overflow flaw in RTSP authentication handling due to improper validation of Authorization header field lengths, which can be triggered by a crafted authentication request. Successful exploitation causes the affected RTSP core service process to crash and triggers an automatic system reboot, resulting in a denial of service (DoS) condition. |
| Reolink Door Bell | 3.0.0.4662_2503122283 | CVE-2025-55637 | Reolink Smart 2K+ Plug-in Wi-Fi Video Doorbell with Chime - firmware v3.0.0.4662_2503122283 was discovered to contain a command injection vulnerability via the setddns_pip_system() function |
| TP L530e | L530e V1.0.0 | CVE-2023-38907 | An issue in TPLink Smart Bulb Tapo series L530 before 1.2.4, L510E before 1.1.0, L630 before 1.0.4, P100 before 1.5.0, and Tapo Application 2.8.14 allows a remote attacker to replay old messages encrypted with a still valid session key |
| Meross MSG100 | <V3.2.3 | CVE-2021-35067 | Meross MSG100 devices before 3.2.3 allow an attacker to replay the same data or similar data (e.g., an attacker who sniffs a Close message can transmit an acceptable Open message). |
| Amcrest | Amcrest IP2M-841B-V3.2.1 | CVE-2020-5735 | Amcrest cameras and NVR are vulnerable to a stack-based buffer overflow over port 37777. An authenticated remote attacker can abuse this issue to crash the device and possibly execute arbitrary code. |
| Amcrest | Amcrest IP2M-841B-V3.2.1 | CVE-2020-5736 | Amcrest cameras and NVR are vulnerable to a null pointer dereference over port 37777. An authenticated remote attacker can abuse this issue to crash the device. |
| Home Assistant Shelly Integration | HA Core <2026.5.4 | CVE-2026-64823 | Home Assistant Core before 2026.5.4 contains a cross-site scripting vulnerability in the Shelly integration's async_get_media_image() method that allows attackers controlling a Shelly device's thumb field to serve arbitrary HTML content by supplying a data URI with a text/html content type without validation against an image-only allowlist |